Use Vivado to build an Embedded System

Lablv-10122510357-金华俊

Introduction

This lab guides you through the process of using Vivado to create a simple ARM Cortex-A9 based processor design targeting the ZYBO board.  You will use Vivado to create the hardware system and SDK (Software Development Kit) to create an example application to verify the hardware functionality.

Objectives

After completing this lab, you will be able to:

Create a Vivado project for a Zynq system

Use the IP Integrator to create a hardware system

Use SDK to create a standard memory test project

Run the test application on the board

Procedure

This lab is separated into steps that consist of general overview statements that provide information on the detailed instructions that follow. Follow these detailed instructions to progress through the lab.

This lab comprises 5 primary steps: You will create a top-level project using Vivado, create the processor system using the Vivado IP Integrator, generate the top-level HDL and export the design to SDK, create a Memory Test application in SDK, and finally, test in hardware.

Design Description

The purpose of the lab exercises is to walk you through a complete hardware and software processor system design. Each lab will build upon the previous lab. The following diagram represents the completed design (Figure 1).

Figure 1. Completed Design

In this lab, you will use IP Integrator to create a processing system based design consisting of the following (Figure 2):

- ARM Cortex A9 core (PS)

- UART for serial communication

- DDR3 controller for external DDR3_SDRAM memory

Figure 2. Processor Design of this Lab

General Flow for this Lab

Step 5:

Verify Functionality in Hardware

Step 4:

Create a Memory TestApp in SDK

Step 3: Generate Top-Level HDL and Export to SDK

Step 2: Create Processor System using IP Integrator

Step 1: Create a project using Vivado

Create a Vivado Project	Step 1

Launch Vivado and create an empty project targeting the ZYBO (having xc7z010clg400-1 device) and using the VHDL language.

Open Vivado by selecting Start > All Programs > Xilinx Design Tools > Vivado 2013.4 > Vivado 2013.4

Click Create New Project to start the wizard. You will see the Create a New Vivado Project dialog box. Click Next.

Click the Browse button of the Project Location field of the New Project form, browse to c:\xup\embedded\labs, and click Select.

Enter lab1 in the Project Name field.  Make sure that the Create Project Subdirectory box is checked.  Click Next.

Figure 3. Project Name Entry

Select RTL Project in the Project Type form, and click Next.

Select Verilog as the Target language and Mixed as the Simulator language in the Add Sources form, and click Next.

Figure 4. Add sources to new project

Click Next two more times to skip Adding Existing IP and Add Constraints

In the Default Part form, select Parts, and using various filters shown in the figure below, select xc7z010clg400-1 part as it is on the ZYBO board. Click Next.

Figure 5. Boards and Parts Selection

Check the Project Summary and click Finish to create an empty Vivado project.

Creating the System Using the IP Integrator	Step 2

Use the IP Integrator to create a new Block Design, add the ZYNQ processing system block, and import the provided xml file for the ZYBO board.

In the Flow Navigator, click Create Block Design under IP Integrator

Figure 6. Create IP Integrator Block Diagram

Enter system for the design name and click OK

Figure 7. Create New Block Diagram

IP from the catalog can be added in different ways. Click on Add IP in the message at the top of the Diagram panel, or click the Add IP icon  in the block diagram side bar, press Ctrl + I,  or right-click anywhere in the Diagram workspace and select Add IP

Figure 8. Add IP to Block Diagram

Once the IP Catalog is open, type “z” into the Search bar, find and double click on ZYNQ7 Processing System entry, or click on the entry and hit the Enter key to add it to the design.

Figure 9. Add Zynq block to the design

Notice the message at the top of the Diagram window that Designer Assistance available.

Double-click on the added block to open its Customization window.

Figure 10. Default configuration form with no peripheral selected

手动设置UART1连接.

Notice now the Customization window shows selected peripherals (with tick marks).

Figure 13. Imported peripherals settings

Click OK to close the Customization window for now.

Configure the processing block with just UART 1 peripheral enabled.

Click on Run Block Automation and select /processing_system7_1

Figure 104. Designer Assistance message

Click OK when prompted to run automation

Figure 15. Run Block Automation

Once Block Automation has been complete, notice that ports have been automatically added for the DDR and Fixed IO, and some additional ports are now visible. The imported configuration for the Zynq related to the ZYBO board has been applied which will now be modified.

Figure 16.11 Zynq Block with DDR and Fixed IO ports

In the block diagram, double click on the Zynq block to open the Customization window for the Zynq processing system.

A block diagram of the Zynq should now be open, showing various configurable blocks of the Processing System.

At this stage, the designer can click on various configurable blocks (highlighted in green) and change the system configuration.

Only the UART is required for this lab, so all other peripherals will be deselected.

Click on one of the peripherals (in green) in the IOP Peripherals block, or select the MIO Configuration tab on the left to open the configuration form

Expand I/O peripherals if necessary, and deselect all the I/O peripherals except UART 1.

- i.e. Remove: 	ENET 0

- USB 0

- SD 0

- Expand Memory Interfaces to deselect Quad SPI Flash

- Expand Application Processor Unit to disable Timer 0.

Figure 17. Selecting only UART 1

Select the PS-PL Configuration tab on the left. Expand GP Master AXI interface and deselect M AXI GP0 interface. Expand General > Enable Clock Resets and deselect the FCLK_RESET0_N option.

Select the Clock Configuration tab on the left. Expand the PL Fabric Clocks and deselect the FCLK_CLK0 option and click OK.

Click on the   (Regenerate Layout) button and see the following block diagram.

Figure 18. Updated Zynq Block

Click on the  (Validate Design) button and make sure that there are no errors.

Generate Top-Level and Export to SDK	Step 3

Generate IP Integrator Outputs, the top-level HDL, and start SDK by exporting the hardware.

In the sources panel, right-click on system.bd, and select Generate Output Products … and click Generate to generate the Implementation, Simulation and Synthesis files for the design

Right-click again on system.bd, and select Create HDL Wrapper… to generate the top-level VHDL model. Leave the Let Vivado manager wrapper and auto-update option selected, and click OK

The system_wrapper.vhd file will be created and added to the project.  Double-click on the file to see the content in the Auxiliary pane.

Figure 19. The HDL Wrapper file generated and added to the project

Notice that the VHDL file is already Set As the Top module in the design, indicated by the icon

You should have the block design open before you export the design to SDK. If it is closed then open the block design by clicking on the Open Block Design under the IP Integrator sub-menu of the Flow Navigator pane.

Select File > Export > Export hardware for SDK…

The Export Hardware for SDK GUI will be displayed.  Select the Launch SDK box, and ensure that Export Hardware is already selected, and click OK to export to, and launch SDK. (Save the design if prompted.)

Note:  Since we do not have any hardware in Programmable Logic (PL) and hence there is no bitstream to generate, the Include bitstream option is not available.

Figure 20. Exporting to SDK

Click Save when prompted to save the design.

SDK should now be open. If only the Welcome panel is visible, close or minimize this panel to view the Project Explorer and Preview panel. A Hardware platform project has been created, and the hw_platform_0 folder should exist in the Project Explorer panel. The .xml file for the Hardware platform should be open in the preview pane. Double click system.xml to open it if it is not.

Basic information about the hardware configuration of the project can be found in the .xml file, along with the Address maps for the PS systems, and driver information.

Figure 21. SDK C/C++ development view

Generate Memory TestApp in SDK	Step 4

Generate memory test application using one of the standard projects template.

In SDK, select File > New > Application Project

Name the project mem_test, and in the Board Support Package section, leave Create New selected and leave the default name mem_test_bsp and click Next.

Figure 22. Create new SDK application project

Select Memory Tests from the Available Templates window, and click Finish.

Figure 23. Creating Memory Tests C Project

The mem_test project and the board support project mem_test_bsp will be created and visible in the Project Explorer window of SDK, and the two projects will be automatically built. You can monitor the progress in the Console panel.

Expand folders in the Project Explorer view, and observe that there are three projects - hw_platform_0, mem_test_bsp, and mem_test.  The mem_test project is the application that we will use to verify the functionality of the design.  The hw_platform includes the ps7_init function which initializes the PS as part of the first stage bootloader, and mem_test_bsp is the board support package.

Figure 24. The Project Explore view

Open the memorytest.c file in the mem_test project (under src), and examine the contents.  This file calls the functions to test the memory.

Test in Hardware	Step 5

Make sure that the JP7 is set to select USB power. Connect the board with a micro-usb cable and power it ON. Establish the serial communication using SDK’s Terminal tab.

Make sure that the JP7 is set to select USB power.

Make sure that a micro-USB cable is connected to the JTAG PROG connector (next to the power supply connector). Turn ON the power.

Select the  tab.  If it is not visible then select Window > Show view > Terminal.

Click on   and if required, select appropriate COM port (depends on your computer), and configure it with the parameters as shown.

此处波特率应该改为172800

Figure 25. SDK Terminal Settings

Run the mem_test application and verify the functionality.

In SDK, select the mem_test project in Project Explorer, right-click and select Run As > Launch on Hardware (GDB) to download the application, execute ps7_init, and execute mem_test.elf.

Figure 26. Launch Application

You should see the following output on the Terminal tab.

Figure 27. SDK Terminal Output

Close SDK and Vivado  by selecting File > Exit in each program.

Conclusion

Vivado and the IP Integrator allow base embedded processor systems and applications to be generated very quickly. After the system has been defined, the hardware can be exported and SDK can be invoked from Vivado. Software development is done in SDK which provides several application templates including memory tests.  You verified the operation of the hardware by downloading a test application, executing on the processor, and observing the output in the serial terminal window.